- Robust analysis

%% Review Week_7

- Fact: LQR controller

: gain margin >1/2 , phase margin = 60degree  Robust!!

- Fact : LQR :

- Total system transfer function is of the order of  two times that of the plant

- It is proved not robust to LQR

%% Kim’s comment

Gain margin, Phase margin from Nyquist

Gω=MωejPω

Nyquist stability criteria :

1+Gω=0

One simple application

- Gain margin = 10, then for constant gain k, so that kG(s) is stable in the closed loop system.

- Phase margin = 60 degree, for time delay e-τs, which has unity magnitude but phase delay as τω. Hence for any time delay τa in forward of G(ω), such that

τaω<60,

Then e-τasG(s)  is stable in the close loop system

%%%

- The fundamental limitation on the controlled system

Sensitivity and complementary sensitivity function

ys=GsKs1+GsKsrs-ns+11+GsKsds
=Tsrs-ns+Ssds

S(s) : Sensitivity function

T(s): Complementary sensitivity function

- The fundamental limitation on the control

Ts+Ss=1, ∀ Gs, Ks :

- Control Objective 
1) in the low frequency range

Reduce the model error and the disturbance effect, good command following,

GωKω≫M

2) in the high frequency range

Reduce the noise effect, 
GωKω≪M

What is the meaning of the above inequality in MIMO? , there is a tool Singular Value Decomposition, H-infinity controller

%%%%

- Robust Control [1]

- Model ;

Consider the following block diagram

- Transfer functions

y=GKwr-y-wn+Gdwd

- y=I+GK-1GKwr-I+GK-1GK wn+I+GK-1Gdwd

- y=TSwr-Tswn+SsGdwd

- Terminology

  - Loop transfer function: the open loop transfer function

Ls=GsKs

  - Tracking error

  - Tracking Error

es=wr-y=Swr+Twn-SGdwd

- Objective : for good performance S(s) is small

- S  is small in frequency range

  - H-infinity norm

- In time domain

xt1=x(t),  xt2= xt2dt

- In frequency domain

S(ω)2= SωS*(ω)dω,  S(ω)∞=maxS(ω

- H-infinity norm of Sω= S(ω)∞

- In scalar, it is the peak value in Bode magnitude plot.

- Singular values of Sω

- In Matrix case for all ω∈[0 , ∞)

Singular values of S=e-values (SωSω*) =[σM, …, σm,…]

%%% Bode plot  it’s domain is frequency

Ex.  Gs=1s2+2s+10 , Ks=1

Bode plot of S(s) , T(s)

%%%

  - Controller objective to minimize the tracking error without exogenous inputs wd, wn.  Find Ks s.t. the magintude Ss= 11+GK∞ is small

Contradiction :  Ts+Ss=1

  - All real systems will have a range of frequencies where S>1

- For any controller, the tracking error is degraded in this range!

- Not only the tracking error, the stability will be degraded by Nyquist analysis

- Loop shaping design / H-infinity design / loop transfer Recovery

- MIMO uncertainty problem [3]

- MIMO

In the SISO case, a good tool to check the stability due to model uncertainty is Nyquist plot( combined Bode plot).The measurements of the robustness are gain margin and phase margin. Let us see the gain margin  Mω  and phase margin Pω.mega phase margin  margin and  n, 
.  the model uncertainty

In order to insure the stability with model uncertainty as, multiplicative uncertainty 
Ls,  G*1+Ls ,

If L(ω)≤ MωejPω  ,

then the closed loop system with the open loop transfer function Gs is stable.

Hence, at least in frequency domain, the Nyquist is a good tool to insure the stability with a model uncertainty

However in the MIMO case, it is difficult to apply Nyquist method. In this case, we may introduce H-infinity norm

%%% Kim’s comment

For the robust, in the time analysis there is no method (?? If my memory is correct). I think there is a Kalman conjecture for stability condition which is still not proved.       %%%

%%% kalman’s conjecture [3]

Consider a system with one scalar nonlinearity

dxdt=Px+qfe,  e=rTs , x∈Rn

Where

P is a stable matrix, q, r constants with appropriate dimemsion, f(e) is a scalar, f0=0

Suppose

k1<dfdt<k2

Then

xkt=Pxk+qke,   k∈(k1, k2)

is stable

But there is a counter example to his conjecture.

- Aizerman’s Conjecture (before Kalman’s conjecture)

Suppose that the nonlinearity f is sector bounded, meaning that for some real k1, k2 with k1<k2, the function satisfies

k1<fee<k-2 , ∀e≠0.

However there are counter examples to Aizermans’ conjecture.

In general, eigenvalues of a matrix is not continuous(robust?) with respect to the variation of parameters.

However, in the frequency domain, bode plot is continuous w.r.t ω.(Bode plot is only in linear system until now) So in general to study the parameter variations we may deal it in the frequency domain.

%%%

- Block Diagram in MIMO

- Definition

- Sensitivity function , complementary function

Ss=I+GsKs-1
Ts= I+GsKs-1GsKs

- The basic tool

Consider

Y jω=GjωUjω

Then

σGjωUjω2≤ Yjω2≤ σ(Gjω)Ujω2

- Control Objectives

  - Sensitivity of modelling

Let us assume

Gs=G*s+ ∆G

- Open loop system

∆Y = ∆GsG*s-1Ys

- Closed loop system

∆Y = I+GsKs-1∆GsG*s-1Ys

- Closed loop system analysis

To small output variance due to the uncertainty

First

Ss= I +GsYs-1 is small

- σ(S): to be small

- Since σ(A)=1σA-1,  σI+GK is large

- Since σA-1≤ σI+A≤ σ(A) +1 , σ(GK) should be large

In conclusion :

σGK≥pω

where p(ω) , the desired level in the interest frequencies, is large in the low frequencies to ensure the little sensitivity in that frequency range.

  - Command following

- As in SISO case, assume D=N=0, then

Ys=TsR(s)

- To follow the command, Ts ~~1

- Ts+S(s)=I, Ss is small

σGK≥pω

  - Disturbance rejection

As in SISO case, Ss is small, implies

σGK≥pω

  - Noise attenuation

As in the case of SISO, T(s) is small

- σT≤ σSσGK==>  σ(GK)/(σI+GK

[1] Brunton, “Data-Drive Science and Engineering”.Chapter 8.8

[2] S. Skogestad and I. Postlethwaite. Multivariable Feedback Control. Wiley, Chichester, 1996.
[3] https://en.wikipedia.org/wiki/Kalman%27s_conjecture